PrimeSafe FSP Template

# FOOD SAFETY PROGRAM

RETAIL MEAT PREMISES

[Insert Business name here]

Conditions of Use for PrimeSafe Assistance Materials:

PrimeSafe’s guidance documents, including the Food Safety Program Template and Industry-Specific Guidelines, are provided for general informational purposes only. PrimeSafe disclaims any liability for any loss or injury directly or indirectly sustained as a result of reliance on these documents. Businesses must not assume that these guidance materials address all potential food safety hazards relevant to their operations.

If using these documents to develop your Food Safety Program, you must edit to tailor them to suit your specific business, products, market requirements, legislative requirements ensuring that all potential food safety hazards are identified and effectively controlled.

It is strongly recommended that you seek independent food safety advice via external consultants regarding your obligations under the applicable regulatory requirements.

© PrimeSafe (2025)

This work, Food Safety Program, Retail Meat Premises, is licensed under a Creative Commons Attribution 4.0 licence. You are free to re-use the work under that licence, on the condition that you credit PrimeSafe as author, indicate if changes were made and comply with the other licence terms. This licence does not apply to any branding, which must not be reproduced.

Content Page

FOOD SAFETY PROGRAM	1

1.	FOOD SAFETY PLAN	4

1.1 Amendment Register	5

1.2 Introduction	6

1.3 Quality Policy Statement	6

1.4 Scope and Purpose	7

1.5 Facility Floor Plan	8

1.6 Organisation	9

1.7 HACCP Team	10

1.8 Product Description & Intended Use	11

1.9 Process Flow Chart	12

1.10 Hazard Analysis	13

1.11 HACCP Audit Table	15

2.	PREREQUISITE PROGRAMS	19

2.1 Calibration	20

2.2 Maintenance Program	22

2.3 Approved Suppliers	23

2.4 Personnel Hygiene	25

2.5 Premises Hygiene	27

2.6 Good Manufacturing Practices	30

2.7 Staff Training	35

2.8 Cleaning & Sanitation	36

2.9 Pest Control	38

2.10 Customer Complaints	39

2.11 Waste Management	40

2.12 Product Recall	41

2.13 Allergen Management	43

2.14 Product Identification and Traceability	44

2.15 Document Control	46

3.	FORMS & MONITORING	47

Form 1 - Weekly Diary	48

Form 2 - Amendment Register	50

Form 3 - Calibration Register	51

Form 4 - Monthly Maintenance and Repair	52

Form 5 - Approved Suppliers List	54

Form 6 - Staff Training Register	55

Form 7 - Induction Training	56

Form 8 - Cleaning & Sanitation Schedule	57

Form 9 – Customer Complaints	58

Form 10 – Customer Complaint Log	59

Form 11 – Meat Transport Vehicle (MTV) Monitoring and Maintenance	60

Form 12.1 – Labelling Assessment for Packaged Meat Products	61

Form 12.2 – Labelling Assessment for Packaged Meat Products with Added Ingredients	61

Form 13 – Declaration and Use of Preservatives	62

4.	APPENDIX	63

Appendix 1 - Calibration	64

Appendix 2 – Pest Control Station Map	65

Appendix 3 – Determining food safety risk – CCP Decision Tree	66

Appendix 4 - Material Safety Data Sheets (MSDS)	67

# FOOD SAFETY PLAN

## 1.1 Amendment Register

| Date | Version | Document | Amendment | Signature |
| --- | --- | --- | --- | --- |
|  |  |  | Ensure that the food safety program version is current, if changes or amendments are made on this document. The version should be updated and recorded. |  |
|  |  |  | [Insert more rows] |  |

In this section, provide a general introduction to your business and outline management’s commitment to the development and implementation of a Food Safety Program. This may include:

- Nature of the business – A brief description of the company’s operations and industry sector.

- Scope of products – The types of food products manufactured, processed, stored, or distributed.

- Major customers – Key markets and customer segments served by the business.

- Commitment to quality and food safety – A statement from management emphasizing the company’s dedication to maintaining high food safety and quality standards.

- Organisational structure – A list or description outlining roles and responsibilities related to food safety within the organisation.

- Ensure the food safety program reference with the relevant legislation and standards.

For further guidance, refer to:

PrimeSafe website – Food Safety programs

1.2 Introduction

## 1.3 Quality Policy Statement

Example: Our business is committed to producing safe, quality products that conform to all state regulatory requirements and meet the highest level of food safety practices. We accomplish this by using a risk-based assessment tool, Hazard Analysis and Critical Control Point (HACCP) to analyse the food safety risks throughout processing and up to the point of sale.

As part of our commitment to food safety and good manufacturing practices (GMP), we ensure our staff are adequately trained to deliver the required food safety outcomes.

For maintenance of the integrity of the food safety program, the instructions contained in it are reviewed internally by the HACCP Team or a PrimeSafe approved auditor. Any amendments to the program are documented and controlled through an amendment register.

Licensee Name:				______________________________________

Meat Processing Facility Location:		______________________________________

Operator:					______________________________________

Signed:					______________________________________

Date:						_____________________________________

In this section document the scope and purpose of the food safety plan.

Refer to: A guide to the implementation and auditing of HACCP – SCARM Report 60

1.4 Scope and Purpose

The food safety program is prepared to ensure that the regulatory requirements of the Victorian Standard for the Hygienic Production of Meat at Retail Premises (2006) and Australian Standard for the Hygienic Production and Transportation of Meat and Meat Products for Human Consumption (AS4696), Food Standards Australia New Zealand are being complied with.

Note: there are other regulatory requirements that will need to be included in this section, please see list below.

Victorian Legislative
-Meat Industry Act 1993
-Meat Industry Regulations 2025
-Food Act 1984

National standards and guidelines
-A guide to the implementation and auditing of HACCP (SCARM Report 60)
-Australia New Zealand Food Standards Code – Labelling requirements
-Australian Standard AS 4696

Victorian standards and guidelines
-Victorian Standard for Hygienic Production of Meat at Retail Premises (March 2006)
-Prime Safe Technical Guideline – Shelf life and labelling

The overall goal of Victorian and Australian Standards are to provide a framework for the production of safe and wholesome meat and meat products. Outcomes required from the application of these Standards are based on the following food safety criteria:

- Microbiological safety; and

- Prevention of physical and chemical contamination.

The Standards described above has been used in conjunction with the Australia New Zealand Food Standards Code as the Code covers matters relating to food safety and public health.

The approval of the food safety program, construction and equipment at this facility have been granted through an audit and review process. Any changes to the program, construction or equipment have been documented and reviewed by PrimeSafe or a PrimeSafe approved auditor.

The food safety program shall remain at this premises where meat and meat products are being processed and/or stored

## 1.5 Facility Floor Plan

In this section document the facility floor plan to show all entry and exits and design of floor plan of how product flows.

For further guidance, refer to:

PrimeSafe website – Managing Food Safety Risks

## 1.6 Organisation Structure

This list comprises all individuals who work within this business

| POSITION IN THE BUSINESS | NAME |
| --- | --- |
| Operator |  |
| [Insert more rows if required] |  |

## 1.7 HACCP Team

This section should list the individuals responsible for reviewing the food safety program and identifying potential food safety hazards with

| ROLE IN THE TEAM | NAME | POSITION IN THE BUSINESS/ EXPERIENCE |
| --- | --- | --- |
| Team Leader |  | In this section list the staff that is on the HACCP team. The team listed is responsible for developing and maintaining this food safety program, updating the HACCP manual, analysing and improving procedures and implementing effective controls to manage food safety risks. Note: You can add more rows to include other role in your HACCP team Refer to: A guide to the implementation and auditing of HACCP – SCARM Report 60 |
| [Insert more rows] |  |  |

Describe the food description and what is the intended use. E.g., Cooked Smoked Ham, consumed chilled or chilled raw meats, cooked prior to consumption.

1.8 Product Description & Intended Use

| Product Description |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Composition ( Tick ✔ ) | Beef |  | Pork |  |  |  | Lamb |  |  |  | Chicken |  |  |  | Other Species (specify) |  |
| Method of Preservation ( Tick ✔ ) | Chilling |  | Freezing |  |  |  | Cooking |  |  |  | Marinating |  |  |  | Other specify |  |
| Packaging – Primary ( Tick ✔ ) | Tray |  | Shrink Wrap |  |  |  | Vacuum Packed |  |  |  | Plastic Bag |  |  |  | Other specify |  |
| Packaging – Secondary (specify) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Storage Conditions | Sell, Store 5°C or below |  |  |  | Sell, Store Frozen |  |  |  |  |  |  |  |  |  |  |  |
| Distribution Method (specify) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Shelf Life (specify) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Labelling Requirements (specify) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Customer Requirements (specify) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sensitive Customer | Infants, Elderly & Immunocompromised |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Final Customer Preparation | Cooked / Heated before consumption |  |  |  |  |  |  |  | Ready to Eat (RTE) |  |  |  |  |  |  |  |

In this section complete details for each product, you produce and describe the intended use (the example from – A guide to the implementation and auditing of HACCP, SCARM Report 60 can be used as a guide).

In this section construct a Process flow chart for each product or like-products by providing a simple, clear description of the steps involved in your process. You can refer to:

A guide to the implementation and auditing of HACCP, SCARM Report 60

1.9 Process Flow Chart

Example:  Retail Butcher Shop – Chilled and Frozen Meat, Manufactured Meat and Packaged Meat Products for Retail Sale

1. Chilled (≤5°C) or Frozen Foods Receival

2. Dry Ingredients Receival (Ambient)

Chilled & frozen meat receival

Less than 7°C for carcass or side, quarter bone in meat;

5°C or below for all other meat

Or frozen

or frozen

Step 1: CCP

Receival Meat &

Dry Ingredients

1. Storage of Chilled (≤5°C) or Frozen Products

2. Storage of Dry Ingredients (Ambient)

Chilled & frozen meat storage

Less than 7°C for carcass or side, quarter bone in meat

5°C or below for all other meat

Or frozen

or frozen

Step 2: CCP

Storage

Weighing, Marinating

Blending

(Optional)

Thawing (temperature)

Trimming, Deboning, Mincing, Cutting etc.

Step 3: CCP

Processing

Step 4: CP

Marinating

Blending

Packaging & Labels

Retail Display & Sale

5°C or below

Or Frozen

Packaging & Labelling (Optional)

Step 5: CP

Retail Display & Sale

Step 6: CCP

Packaging & Labelling

Storage

5°C or below

Or frozen

Transport (Optional)

5°C or below

Or Frozen

Step 7: CCP

Storage

Step 8: CCP

Transport

When developing a process flow chart, please ensure that you verify the process to confirm it reflects the current process by annual review.

In the Hazard Analysis, list all potential hazards associated with each step. You will need to conduct a hazard analysis and consider any control measures to mitigate these hazards. Examples 1 and 2 illustrate common meat processing activities typically conducted by retail butchers.

This section requires the food business to review and assess.

## 1.10 Hazard Analysis

Example 1: Chilled Meat/Products for Retail Sale

| Step | CCP | Hazard¹ | Control Measure |
| --- | --- | --- | --- |
| 1. Receival | CCP 1 | M: Growth of pathogenic microorganisms C: - P: Foreign material | Reject meat and meat products that is not within temperature specification (Less than 7ºC for carcass, side, quarter, or bone in meat and 5ºC or below for all other meat). Frozen products are hard frozen. |
| 2. Storage | CCP 2 | M: Growth of pathogenic microorganisms C: - P: - | Less than 7ºC for carcass, side, quarter, or bone in meat and 5ºC or below for all other meat. Frozen products are hard frozen. |
| 3. Processing | CCP 3 | M: Growth or contamination of pathogenic microorganisms C: - P: Foreign material | Maintain temperature of product at 5ºC or below Mincing: Bone fragments are no greater than 7mm Foreign material: wood, plastic, and metal. |
| [insert steps] | [insert CCP] | [insert hazard] | [insert control measure] |
|  |  | ¹ M = Microbiological C = Chemical and allergens P = Physical |  |

1.10 Hazard Analysis

Example 2: Chilled Meat, Manufactured Meat and Packaged Meat Products for Retail Sale

| Step | CCP | Hazard¹ | Control Measure |
| --- | --- | --- | --- |
| 1. Receival | CCP 1 | M: Growth of pathogenic microorganisms C: - P: Foreign material | Reject meat and meat products that is not within temperature specification (Less than 7ºC for carcass, side, quarter, or bone in meat and 5ºC or below for all other meat and chilled products.) Frozen products are hard frozen. |
| 2. Storage | CCP 2 | M: Growth of pathogenic microorganisms C: - P: - | Less than 7ºC for carcass, side, quarter, or bone in meat and 5ºC or below for all other meat and chilled products. Frozen products are hard frozen. |
| 3. Processing | CCP 3 | M: Growth or contamination of pathogenic microorganisms C: - P: Foreign material | Maintain temperature of product at 5ºC or below Mincing: Bone fragments are no greater than 7mm Foreign material: wood, plastic, and metal. |
| 4. Marinating and Blending | CP | M: - C: Cross contamination of allergens P: Foreign material | All allergen and allergen-containing products must be segregated from non-allergenic product. Follow Allergen Management Procedure 2.15. Foreign material: wood, plastic, and metal. |
| 5. Retail Display & Sale ( | CCP 4 | M: Growth of pathogenic microorganisms C: - P: - | Maintain temperature at 5ºC or below. Frozen products are hard frozen. Gloves, utensils, or inverted bag method used when handling meat and meat products. |
| [Insert step] | [insert CCP] | [Insert hazards] | [Insert control measure] |
|  |  | ¹ M = Microbiological C = Chemical P = Physical Q = Quality |  |

Determine your CCPs and establish critical limits for each CCP.

Please refer to steps 6 and 7: list potential hazards, conduct a hazard analysis, and identify the CCPs. For guidance, see 'A Guide to the Implementation and Auditing of HACCP' and Attachment A: CCP Decision Tree or Appendix 3.

## 1.11 HACCP Audit Table

Example 1(a) Chilled Meat/Products for Retail Sale

| Step | Hazard | Control Measure | Critical Limit | Monitoring | Corrective Actions | Records | Verification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Receival (CCP) | Growth of pathogenic microorganisms | Temperature control | Less than 7ºC for carcass, side, quarter, or bone in meat 5ºC or below for all other meat The HACCP audit table should establish the following: - Critical limits for each CCP, as per the Australian Standard where applicable. - A monitoring system for each CCP. - A corrective action plan for any CCP deviations that may occur. - A list of records used to support the control of CCPs. - Verification of these records. Frozen products are hard frozen | What: Temperature How: Thermometer Where: Receival Dock When: Every delivery Who: Operator | Where the temperature is warmer than 5°C for meat products, then provided the temperature is no warmer than 7°C accept the product but immediately place it under active refrigeration. Contact the approved supplier and raise a Corrective Action Request (CAR). For carcass, side and bone-in product do a risk assessment (e.g. temperature and source of stock) before accepting it; Raise a CAR. Assess frozen products and if they are no longer frozen, then they must be placed in the cool room and not the freezer (thawed product). | Form 1 – Weekly Diary | Weekly review of Form 1 – Weekly Diary. Review Form 5 - Approved Suppliers List, annually. Check Form 3 - Calibration Register for the thermometer, quarterly. |
| 2. Storage (CCP) | Growth of pathogenic microorganisms | Temperature control | Less than 7ºC for carcass, side, quarter, or bone in meat 5ºC or below for all other meat Frozen products are hard frozen | What: Temperature How: Temperature Gauge Where: Chiller / Freezer When: Twice Daily Who: Operator | Re-calibrate the temperature gauge. Contact a refrigeration mechanic. Discard product if temperature is above 7°C. | Form 1 – Weekly Diary Form 5 - Calibration Register | Weekly review of temperatures on Form 1 – Weekly Diary. Check Form 3 - Calibration Register, readings for chiller and freezer gauges, quarterly. |
| 3. Processing (CCP) | Growth of pathogenic microorganisms | Temperature control | 5ºC or below | What: In-process meat product How: Temperature Gauge Where: Processing Area When: Twice Daily Who: Operator | Temperatures must be 5°C or below, if not place under refrigeration immediately. | Form 1 - Weekly Diary Form 3 - Calibration Register | Weekly review of temperatures on Form 1 – Weekly Diary. Check Form 3 - Calibration Register for the thermometer, quarterly. |
| [Insert Step] | [Insert Hazard] | [Insert Control Measure] | [Insert Critical Limit] | [Insert Monitoring] | [Insert Corrective Actions] | [Insert Records] | [Insert Verification] |

1.11 HACCP Audit Table

Example 2(a) Chilled Meat, Manufactured Meat and Packaged Meat Products for Retail Sale

| Step | Hazard | Control Measure | Critical Limit | Monitoring | Corrective Actions | Records | Verification |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Receival (CCP) | Growth of pathogenic microorganisms | Temperature control | Less than 7ºC for carcass, side, quarter, or bone in meat. 5ºC or below for all other meat and chilled products. Frozen products are hard frozen. | What: Temperature How: Thermometer Where: Receival Dock When: Every delivery Who: Operator | Where the temperature is warmer than 5°C for meat products, then provided the temperature is no warmer than 7°C accept the product but immediately place it under active refrigeration. Contact the approved supplier and raise a Corrective Action Request (CAR). For carcass, side and bone-in product do a risk assessment (e.g. temperature and source of stock) before accepting it; Raise a CAR. Assess frozen products and if they are no longer frozen, then they must be placed in the cool room and not the freezer (thawed product). | Form 1 - Weekly Diary | Weekly review of Form 1 – Weekly Diary. Review Form 5 - Approved Suppliers List, annually. Check Form 3 - Calibration Register for the thermometer quarterly. |
| 2. Storage (CCP) | Growth of pathogenic microorganisms | Temperature control | Less than 7ºC for carcass, side, quarter, or bone in meat. 5ºC or below for all other meat and chilled products. Frozen products are hard frozen. | What: Temperature How: Temperature Gauge Where: Chiller / Freezer When: Twice Daily Who: Operator | Re-calibrate the temperature gauge. Contact a refrigeration mechanic. Discard product if temperature is above 7°C. | Form 1 – Weekly Diary Form 3 - Calibration Register | Weekly review of temperatures on Form 1 – Weekly Diary Check Form 3 - Calibration Register, readings for chiller and freezer gauges, quarterly. |
| 3. Processing (CCP) | Growth of pathogenic microorganisms | Temperature control | 5ºC or below | What: In-process meat product How: Temperature Gauge Where: Processing Area When: Twice Daily Who: Operator | Temperatures must be 5°C or below, if not place under refrigeration immediately. | Form 1 - Weekly Diary Form 3 - Calibration Register | Weekly review of temperatures on Form 1 – Weekly Diary. Check Form 3 - Calibration Register for the thermometer, quarterly. |
| 4. Marinating and Blending (CP) | Cross contamination of non-allergenic products with allergenic products Consumer health risk from additives | Segregation of non-allergenic products from allergenic products. Usage of Sulphur Dioxide (SO2) | No allergens in non-allergenic products. Sausage and sausage meat: 500mg/kg (FSANZ) All other meat must have no SO2. | What: Processing How: Visual inspection Where: Processing Area When: When processed Who: Operator What: Use of SO2 How: Weighing Where: Processing Area When: During processing Who: Operator | Discard product if cross contamination occurs or rework the product into an allergen containing product containing the same allergen/s. Discard product if amount of sulphur dioxide in a product is unknown or send to a NATA approved laboratory for testing to confirm level of SO2. | Form 1 – Weekly Diary Form 13 – Declaration and Use of Preservatives | Weekly review of Form 1 – Weekly Diary. Weekly review of Check Form 13 – Declaration and Use of Preservatives. |
| [Insert Step] | [Insert Hazard] | [Insert Control Measure] | [Insert Critical Limit] | [Insert Monitoring] | [Insert Corrective Actions] | [Insert Records] | [Insert Verification] |

# PREREQUISITE PROGRAMS

In the following sections, document information on policies and procedures for each of the Prerequisite Programs that support the HACCP plan established in your organization.

Refer to 'A Guide to the Implementation and Auditing of HACCP – SCARM Report 60.

## 2.1 Calibration

All equipment used during food processing to conduct food safety system checks must be calibrated and in good working order. Equipment such as thermometers and scales require calibration internally or externally. The Operator/Quality Assurance Manager of the facility is responsible for calibration of all equipment and must complete and review all calibration records. Calibration must also be considered as part of the internal audit process.

In this section, document how calibration is conducted for all equipment on site . Ensure this procedure covers all the equipment used on the premises must be calibrated and maintained in working order.

- Records must be kept. For example, hand-held thermometers should be calibrated monthly and recorded on a Form 3 Calibration Register. If in-house calibration of temperature gauges on coolrooms/freezers occurs, this should also be recorded on the same checklist.

- The Calibration Register records must be kept on file for a minimum of 3 months.

- Thermometer gauges on coolrooms, freezers, and ice rooms should be calibrated regularly, for example, every 3 months, and the calibration results recorded.

- See Appendix 1 for Calibration instructions

The calibration of all equipment is recorded on Form 3 Calibration Register. A sticker with the date of calibration and the variance is placed on the equipment. If during calibration the variance reading is not within specification, it must be communicated to all personnel using the equipment and recorded on Form 4 Monthly Maintenance and Repair to be either repaired or replaced. Re-calibration is required for equipment that has been dropped, damaged, or repaired. Procedures for calibration of scales and thermometers can be found in Appendix 1 Calibration.

Calibration of equipment can occur internally or externally.

Internal and External Calibration

If equipment is calibrated internally then trained staff must do the calibrations and record it in Form 3. If calibrations are done externally then the equipment must be sent to an external company or laboratory for calibration and the records received from the calibrators must be recorded in Form 3 and must be kept together with Form 3.

Scales

Scales must be calibrated every 3 months. Calibration of scales involves testing against certified trade weights. Often this is done internally and recorded in Form 3. Sometimes the calibrator may come on-site to do the calibrations or the scales are sent off-site. In both of these cases the calibrations must be recorded in Form 3. If certified weights are used on-site then they must have current calibration certificates. If the certificates expires, the weights must be sent to an external company certifying the weight for re-certification. New scales will be deemed accurate if the variation from the true weight is less than 0.1% of the capacity of the scale. Scales which have been in operation must not vary more than 0.2% from the test weight.

Thermometer

Hand-held and fixed thermometers must be calibrated every 3 months. The acceptable temperature variance is +/- 1°C.

Hand-held thermometer is calibrated using Ice Water Slurry and Boiling Water Method to ensure thermometer is accurate for upper and lower temperature ranges.

Fixed Thermometer (chillers, refrigerators, and display cabinets) is calibrated with a hand-held thermometer that has been calibrated with Ice Water Slurry Method.

## 2.2 Maintenance Program

The design, working space and layout of the facility and its equipment must be constructed to facilitate the hygienic production and storage of meat and meat products.

The construction requirements of the premises and equipment are outlined in Section 4 of the Victorian Standard for the Hygienic Production of Meat at Retail Premises.

The documented maintenance program must include the following:

- A system to routinely review the building and equipment for any defects that may cause food safety issues (physical, chemical, or biological contamination) with meat and meat products produced or stored at the facility;

- Scheduled monthly maintenance check that documents any non-conformances;

- Any non-conformances must be followed up with corrective actions;

- Where there has been repeated non-conformances then a preventative maintenance program is installed to ensure consistent compliance with the Standard; and

In this section, document information on the procedures for the Maintenance Program established in your organisation.

Ensure that you include the relevant aspects of the premises, equipment and food transport vehicles as part of the program.

An example of a Form 4 Monthly Maintenance and Repair and Form 1 Pre-Operational Checklist can be found in the Forms & Monitoring of this program.

PrimeSafe advised of any major changes to building and equipment will require inspection and approval prior to use.

The condition of the facility and equipment must be inspected at least on a monthly basis and recorded on Form 4 Monthly Maintenance and Repair.

The Meat Transport Vehicle (MTV) is maintained through monthly maintenance in accordance with the Australian Standard for Hygienic Production and Transportation of Meat and Meat Products for Human Consumption (AS4696); Section 25. Any defects or maintenance required is recorded on Form 4 and Form 11 Meat Transport Vehicle (MTV) Monitoring and Maintenance.

The proposed time-frame to rectify any non-conformances is based on the impact the non-conformances may have on food safety, which is determined by the operator. The operator reviews Form 4 every month to verify the completion of maintenance work.

The operator is responsible for the review of the Maintenance Program every 12 months or as required.

## 2.3 Approved Suppliers

All food products, raw materials, packaging materials and chemicals must be purchased from Approved Suppliers. The approved suppliers for the business are listed on Form 5 Approved Suppliers List. The Operator/ Quality Assurance Manager reviews the Approved Suppliers List annually or when a supplier is added or removed from the list.

In this section, document how the organisation approve suppliers on your Approved Supplier Program.

An example guide of an Approved Supplier Program and the required documents is as follows:

- All goods and services must be sourced from an approved supplier.

- It is your responsibility to determine criteria for approving and maintaining suppliers (e.g., HACCP program required for high-risk ingredients).

- All approved suppliers are listed on the Approved Supplier List (Form 5) along with the products they supply.

- Documents provided by the supplier and the completed Supplier Approval Application Form must be kept.

Prior to approving a supplier, the Operator/Quality Assurance Manager reviews the suppliers’ documentations and ensures the supplier meets the following criteria:

- A registered business;

- Has a safety/quality assurance certification (e.g. HACCP accreditation); and

- Provides a product specification.

Suppliers’ certification and documentation are kept on file with the approved supplier list as records.

Receival of all food products, raw materials, packaging materials and chemicals are recorded on Form 1 Weekly Diary.

Prior to accepting packaged meat and meat products, the Operator/ Quality Assurance Manager shall verify that the packaging label contains the following information:

- Name of supplier and business address;

- PrimeSafe licence number (if applicable);

- Description of the product;

- Date of packaging and use by/best before date;

- Batch number; and

- Weight of the product.

For non-meat products, the supplier may provide a Certificate of Analysis (CoA) or Certificate of Compliance (CoC) for all incoming goods to confirm the products meets the product specification. For all chemicals and preservatives (sulphur dioxide, nitrates & nitrites), the Material Safety Data Sheet (MSDS) must be kept by the business and the dosage and any additions to food recorded. All chemicals used by the business must be suitable for use in a   food-processing environment and must not jeopardise the wholesomeness of meat and meat products.

Meat, meat products and/or seafood (not live) must be delivered in a licensed Meat Transport Vehicle where the products and the environment are under controlled temperatures. When the product is not within specification at receival, the Operator/Quality Assurance Manager must issue a corrective action request and notify the approved supplier of the non-conformance. A record of the completed corrective action and investigation completed by the supplier must be kept on file.

If there is continued pattern of non-conformance from the supplier, then a risk assessment is conducted and the supplier shall be removed from Form 5. The Operator/Quality Assurance Manager will source a new supplier and go through the approval process with the new supplier.

## 2.4 Personnel Hygiene

Food handlers have the responsibility to maintain appropriate personnel hygiene practices to minimise the risk to food safety. The following shall be maintained:

- A training program to facilitate hygienic processing of products for human consumption shall be implemented. The training can be recorded on Form 6 Staff Training Register;

- Operators shall ensure that no employee while knowing to be suffering from, or to be a carrier of a disease capable of being transmitted through food, or while afflicted with infected wounds, sores, or diarrhoea, is permitted to work in any capacity in which it is possible either directly or indirectly to contaminate product with pathogenic microorganisms. Any person so afflicted shall report the illness to management;

In this section, document information on the procedures for the Personal Hygiene Program established in your organisation.

Personal hygiene is vital for minimizing the risk of food poisoning or contamination incidents affecting the business. Businesses should use this procedure to train their staff on personal hygiene.

Particular attention shall be paid to infected wounds and sores, enteric infection, and respiratory diseases;

- Every person who is cut or injured, shall cease work until their wound is suitably dressed. No person working in a licensed premises area shall wear any bandage unless that bandage is completely protected by a waterproof and conspicuously coloured covering that cannot be easily detached;

- Every person engaged in the handling of meat and meat products while on duty shall thoroughly wash their hands using liquid sanitiser and potable warm water when potential contamination has occurred;

Food handlers must wash their hands:

- Before commencement or re-commencing of work;

- Before working with cooked/ready-to-eat products after handling of raw food;

- After toilet, cigarette, and lunch break;

- After touching face, body, and hair; and

- After sneezing and coughing.

- Protective gloves for use in the handling of meat shall be maintained in a sound clean and sanitary condition;

- Every person involved in processing of meat and meat products fit for human consumption shall maintain a high standard of personal cleanliness. They shall at all times wear suitable clothing of a colour that allows detection of visible contamination, head covering enclosing hair and enclosed footwear. These articles shall be kept clean and consistent with the work in which the person is engaged;

- Protective clothing such as aprons, gloves and implements shall be properly cleaned at the end of the day and when necessary in a facility provided for this purpose;

- Aprons and personal working equipment including protective gloves, knives and scabbards shall not be worn into toilet areas. Hooks shall be provided at designated sites within the production areas for the hanging of these articles; and

- Any behaviour, which could result in contamination of meat, such as eating, smoking, spitting, and throwing of product, is prohibited.

## 2.5 Premises Hygiene

The hygiene of the premises, equipment and services is managed to ensure that the wholesomeness of meat and meat products is not jeopardised.

In this section document information on the Premise Hygiene Program established in your organisation.

Premise hygiene is a vital part of minimising the risk of food poisoning or a contamination incident affecting the business.

The procedures should reflect the requirements of the relevant standard – Retail butcher shop -(Victorian Standard for Hygiene Production of Meat at Retail Premise, AS 4696 and Food Standards Code for processing high risk meat products such as Smallgoods).

Facilities at this premises will allow cleaning and sanitising of the premises, equipment, and clothing.

Hand-washing facilities

- Provided with hot and cold water from a central outlet;

- Provided with an effective sanitising agent;

- Equipped with taps that are not operated by hand;

- Provided with hand drying facilities that do not contaminate washed hands or the surrounding area;

- Connected directly to a drainage system; and

- Of a size that allows easy and effective handwashing;

Equipment

- Knife sterilisers and other equipment are provided with potable water no less than 82°C; and

- An overflow is contained and directed into a drainage system.

Storage facilities

Adequate facilities is provided for the storage of:

- All processing and packaging materials;

- Laundered and protective clothing;

- Cleaning and sanitising equipment; and

- Aprons and gear (knives, steels etc.) when not in use.

Chillers and Freezers

- Chiller and freezer capacity is adequate to accommodate the maximum total quantity of product likely to be held on the premises at any one time;

- Facilities for the chilling and storage of carcases and quarters is constructed to enable the surface temperature of carcases to be maintained at no more than 7°C;

- Facilities for the chilling  and storage of meat portions less than quarters is constructed to enable product temperature to be maintained at no more than a maximum of 5°C at the site of microbiological concern;

- If carcasses are removed from the chiller then the surface temperature of the carcasses must be maintained at no more than a maximum of 7°C;

- Facilities for the freezing and storage of meat and meat products is constructed to operate at a temperature to ensure that meat and meat products are hard frozen and maintained in a frozen state;

- Rack and stands are used to ensure products are stored in a hygienic condition; and

- Cooling unit discharge is contained and directed to the drainage system.

Loading Areas

- Is constructed to ensure that the wholesomeness of meat and meat products is not jeopardised.

Drains and Effluent

- Floors are constructed so that liquids do not accumulate on the surface, and are coved at the wall and floor junctions;

- Open drains shall be covered with removable grating;

- Drainage system shall prevent odours, vermin, any objectionable material, or storm water entering the premises;

- Floor drainage directed to a save all or other approved means of removing solids and suspended fats;

- Effluent and sewage shall be disposed of by means required by the relevant authorities and in a manner that prevents contamination to potable water supply and does not jeopardise the wholesomeness of meat and meat products; and

- Wastewater from display cabinets or chiller plates is contained and directed into the drainage system.

Amenities

- Location and construction of amenities does not jeopardise hygienic processing of meat and meat products.

Potable Water

- Only potable water is used that is consistent with the Standards for drinking water in Victoria which is taken to be the Australian Drinking Water Guidelines;

- The Australian Drinking Water Guidelines require monitoring of the microbiological quality of drinking water by testing for E. coli in a 100ml sample of water.  A sample of water should not contain any E. Coli organisms.

- If the facility is on a town water supply, results of water quality must be obtained annually from the relevant water authority (Note: sampling for this purpose is not required at the facility).

- Where a facility is not on town water supply or a facility is on a town water supply that is not potable, at a minimum, PrimeSafe approval will required to use the water. To establish continued potability of the water, E.coli must be tested in 100mL of water on a quarterly basis.

- Recycled water will not be used unless prior approval from PrimeSafe. If recycled water is used then testing for E.coli is based on a risk analysis with testing on a weekly basis as generally accepted.

## 2.6 Good Manufacturing Practices

The business operator shall ensure good manufacturing practices at all times by:

In this section document information on Good Manufacturing Practices (GMP) program established in your organisation.

The procedure for good manufacturing practices conducted at your facility is vital for minimising the risk of food poisoning or food safety contamination incidents affecting the product wholesomeness

The procedure should reflect the requirements of the relevant standards, such as the Victorian Standard for Hygiene Production of Meat at Retail Premises and AS 4696 for processing high-risk meat products like smallgoods

- Maintaining a high standard of hygienic processing as described in the Victorian Standard for the Hygienic Production of Meat at Retail Premises;

- Correcting any insanitary practices when they occur; and

- Ensuring that there are staff readily available for correcting deficiencies.

Receival

During receival, the operator has the responsibility to take all practical measures to ensure that only products within specification are accepted. Prior to accepting deliveries, the operator/ quality assurance manager should:

- Conduct a visual inspection for defects of colour, foreign matter etc.;

- Check and record product temperature on Form 1 Weekly Diary;

- Review Certificate of Analysis (CoA) (if applicable);

- Review laboratory test results (if applicable); and

- For carcass received, check branding (except for poultry).

When receiving potentially hazard foods, the operator must only accept meat, meat products, seafood products and perishable food products at temperatures of:

- 7ºC or below for a carcass side, quarter or bone-in major separated cuts;

- 5ºC or below for any other meat, seafood, or perishable food; and

- Frozen products as frozen to the core (not partially thawed).

All received products (supplier and quantity) must be recorded on Form 1. Temperature of meat, meat products and other potentially hazardous foods are also recorded on Form 1 and placed into storage without delay.

Storage

The following requirements shall be adhered to for storage of all chilled and frozen meat and seafood products:

- Chilled meat and seafood products are stored and maintained at 5°C or below;

- Frozen products must remain frozen until sold or processed;

- Frozen products that are sold as chilled products must indicate that they have been thawed for sale;

- Raw products are stored separately from cooked and to ready-to-eat foods to prevent contamination;

- Where separate storage areas are not available, ready-to-eat products shall be stored above raw food on the racking;

- To prevent contamination, all unpackaged meat and seafood products are stored in covered containers;

- Tubs used to store food products are not in direct contact with the floor and placed on shelving or pallets. All food products are stored off the floor to prevent pests and other contamination;

- The chiller must be operated and maintained to ensure meat and seafood product temperature is maintained at 5ºC or below;

- The refrigeration is not overcrowded to allow chilled air to circulate around the food products;

- Dry products are stored in a clean, dry, well-ventilated, and cool area and rotation of dry products using first in first out (FIFO) method; and

- The temperature of meat and meat products and the environment is monitored daily and recorded on Form 1.

Handling Practices

All food handlers must take all practicable measures to prevent food from being contaminated.

- Gloves, utensils, or inverted bag method can be used when handling food;

- Change of gloves is required between handling of raw and cooked foods;

- Where protective gloves are used in the handling of meat it should be maintained in a sound, clean and sanitary condition;

- Hands are washed thoroughly prior to handling cooked food or ready-to-eat products; and

- After handling of money, wash hands thoroughly before handling unpackaged food.

The temperature of one in-process meat product is recorded on Form 1 twice daily.

Thawing of Meat and Meat Products

- The thawing of meat products must be under controlled conditions of no warmer than 10°C and as follows:

- Carcass, side, quarters, or major bone-in products are no warmer than 7°C;

- All other meat no warmer than 5°C.

- Thawed meat and meat products shall not be re-frozen.

Use of Preservatives

If your premises are processing sausages, you will need to ensure compliance with the Food Standards Code Australia New Zealand for the usage of the preservative Sulphur Dioxide (SO2).

If you have SO2 on site, you must declare it, as it is a known allergen risk.

IMPORTANT: Sulphur dioxide cannot be used on fresh meats.

Food Standards Code Australia New Zealand prescribes the maximum level of sulphur dioxide (SO2) in meat and meat products.

- The following is allowed:

  - Sausage and sausage meat is to have a maximum limit of 500mg/kg SO2; and

  - All other meat and meat products is to have no SO2.

- Sausage premix and sulphur dioxide preservatives are used in accordance to manufacturer’s recommendation;

- Sulphur dioxide must be diluted with water to ensure even distribution within the product; and

- If preservatives are used, record the usage of preservatives on Form 13 Declaration and Use of Preservatives.

Display

- To prevent contamination, meat and seafood products must be enclosed, contained, or wrapped and not allow accessibility for self-service;

- The display units must be checked daily and the temperature of the display units are recorded on Form 1;

- Temperature of two products for retail sale must be recorded twice daily.

- Meat and seafood must be stored 5ºC or below or above 60°C. Food that is displayed must maintain within its temperature specification and food that is intended to be displayed frozen is hard frozen;

- Unpackaged meat and seafood for retail display must contain a sign stating the species of seafood/meat, pricing, and country of original labelling (CoOL); and

- Meat products which have been frozen and thawed for sale must be labelled accordingly

- Frozen meat and meat products are hard frozen.

Packaging

- Packaging materials are food-grade, clean, free from substances capable of imparting any unwholesomeness to the product and stored in an area to prevent contamination by dust, pest, and chemicals; and

- All packaging materials are examined prior to use and any damaged packaging materials are not to be used.

Labelling

- All meat and seafood that is sold as wholesale is required to meet the labelling requirements of the Food Standards Code and Victorian Standard for the Hygienic Production of Meat at Retail Premises; and

- The labelling assessment of packaged meat is recorded on Form 12 Labelling Assessment.

Dispatch

- Meat and meat products must be dispatched at a temperature of 5ºC or below;

- If there is an issue then the product must be kept under temperature until dispatch; and

- The temperature that meat and meat products are dispatched at must be recorded on Form 1.

Transportation

- Meat and seafood (not live) products are transported in a PrimeSafe licensed meat transport vehicle;

- Vehicles with refrigeration have the capacity and sufficient air circulation to maintain temperature of 5ºC or below; and

- The maintenance of the vehicle, temperature of the meat and/or seafood products and temperature of the vehicle is recorded on Form 11 Meat Transport Vehicle (MTV) Monitoring and Maintenance.

Non-conforming Product

- Meat and seafood products that are not safe for human consumption must be condemned and disposed of;

- Condemned meat and seafood products must not be displayed or sold; and

- Condemned meat and seafood products stored for disposal must be immediately marked with ‘HOLD’ and stored separately to products fit for human consumption.

## 2.7 Staff Training

It is essential to conduct staff training to ensure staff understand food safety hazards in order to prevent, minimise and eliminate food safety risk. Employees who undertake food handling operations must have the skills and knowledge to handle food safely.  Internal and external training is recorded on Form 6 Staff Training Register.

The Operator/ Quality Assurance Manager is responsible for ensuring that the training requirements of the business are met. The type of training required must be adequate to fulfil the responsibilities required by the role and must match the job description. If it is determined that staff require further training to perform their duties, then the training can be delivered by a person accredited to deliver that training. In particular, training that covers food safety (i.e. HACCP) and medium risk processing activities such as cooking products to serve hot. This training must be delivered by a person accredited to deliver that training. The person delivering the HACCP training must have acquired certification through a registered training organisation (RTO). The details of all training, including the trainer and RTO must be documented in Form 6. All training certificates must be kept on file.

In this section document information for the Staff Training Program established in your organisation.

All staff must be trained to perform their jobs safely and competently. Training should cover their specific work activities, which may include at least:

- Good personal hygiene

- Food handling procedures

- Cleaning and sanitation

Records of staff training should be maintained for customer complaints and auditing purposes.

Note: This training procedure should include external visitors who attends on site.

All new employees must be given appropriate induction training at the commencement of their employment and should read and understand this Food Safety Program. As part of the induction training, the Operator/Quality Assurance Manager must ensure all new employees have received appropriate training and are competent to carry their roles and responsibilities. The Operator/ Quality Assurance Manager complete all action items on Form 7 Induction Training for all new employees and contractors.

It is the Operator/ Quality Assurance Manager’s responsibility to ensure staff who are responsible for food safety and are handling food, attend refresher training as required to refresh their skills and knowledge in food safety and hygiene matters.

## 2.8 Cleaning & Sanitation

Cleaning and sanitation is conducted routinely to ensure hygienic processing of meat and meat products. The facility including processing areas, chillers, storerooms, amenities, washrooms, and equipment including machines, utensils and tools used on food contact areas are cleaned and sanitised routinely to ensure that they do not jeopardise the wholesomeness of meat and meat products. The environment including walls, ceiling, floors, doors, and all non-contact areas are also routinely cleaned to ensure that meat is processed in a hygienic environment.

The premises and equipment shall be:

- Cleaned before operations commence each day where the pre-operational assessment identifies hygiene issues;

- Cleaned at the end of the day;

- Cleaned and sanitised whenever is necessary to do so to prevent contamination;

- Cleaning equipment is stored in a clean and dry area;

- Staff members must wash their hands and change protective clothing after cleaning;

- Material Safety Data Sheets (MSDS) are available on-site;

- Chemicals are used as stated in the manufacturer’s instructions;

- All chemicals used for cleaning and sanitation are clearly labelled; and

- Stored away from food storage and processing areas.

Cleaning practices must follow a routine to prevent re-contamination e.g. cleaning started at the back and then work towards the front of the facility, cleaning walls and equipment from top to bottom.

In this section, document information for the Cleaning and Sanitation Program established in your organisation.

Ensure that you incorporate the relevant aspects of the premises, equipment, and Meat transport vehicles (MTV) as part of the program.

A Pre-Operational Checklist is a useful tool. This type of check can be used at the commencement of each shift/day to ensure the premises and equipment are satisfactory for use. It can be carried out daily before processing to ensure that all surfaces are clean prior to use.

Cleaning Schedule

The cleaning schedule, which details the cleaning area/item, cleaning frequency and strength of chemicals, is recorded on Form 8 Cleaning and Sanitation Schedule.

Pre-operational Check

The Pre-operational check, which verifies the Cleaning Schedule is recorded on Form 1 Weekly Diary. When areas/surfaces/equipment are non-compliant with the cleaning schedule, a corrective action is raised and cleaning and sanitation is completed at the earliest possible opportunity. If deficiencies in the cleaning and sanitation program affects product food safety then a risk assessment is completed, which may include product being sent for microbiological testing. The operator of the facility or a person responsible for food safety must sign off any corrective actions.

Cleaning Procedures/Instructions

The cleaning procedure involves the following steps:

- The detergent and sanitiser is approved for use in food processing facilities;

- The detergent and sanitiser is used in accordance to manufacturer’s recommended dosage;

- Pre-clean – wipe down or remove food scraps;

- Wash – using hot water and detergent to remove particles, grease, and dirt;

- Rinse – removal of the detergent, loosened dirt, and grease;

- Sanitise – use of a sanitiser to reduce bacteria on the surface;

- Final Rinse – rinse off the sanitiser (not applicable for a no-rinse sanitiser);

- Dry – leave the area to air dry; and

- Inspect – visually inspect the area/equipment.

## 2.9 Pest Control

The Operator/Quality Assurance Manager (QAM) is responsible for Pest Control Inspections of each monitoring station on a weekly basis. Observations of monitoring station inspections and evidence of infestation are recorded on Form 1 Weekly Diary. For any pest sightings or damage to monitoring stations, corrective actions are taken accordingly to ensure zero tolerance of pests.

In this section document information for  the Pest Control Program established in your organisation.

Animals and pests, including insects and rodents, must be excluded from the premises.

For example, any evidence of pests can be recorded on the Pre-Operational Checklist (Form 1)

Monitoring stations are located in areas that will minimise the risk of pests from coming into contact with food or food contact surfaces. To prevent harbourage of pests, the pest inspection involves checking:

- Doors are kept shut when not in use;

- Garbage storage area is clean and tidy;

- Garbage is cleared routinely from production areas and when there is build up;

- Food premises and equipment is clean and well maintained;

- Sealing of holes, gaps and cracks in walls, ceilings, and below external doors; and

- Food products are covered and off the floor.

Monitoring stations are numbered and placed in accordance with the Pest Control Station Map of the facility shown in Appendix 2. Each station is labelled with the date of service. Replacement of monitoring stations or additional monitoring stations that are required are recorded on Form 4 Monthly Maintenance and Repair.

Pest Control monitoring must be conducted monthly by trained personnel internally or by a licensed pest contractor. A Pest Control Contractor shall visit the facility on a monthly basis or as required. Where there is presence of pests (insects and rodents) in a monitoring station or additional monitoring stations are required, the Operator/QAM should consider contacting a Pest Control Contractor. The Operator/QAM must ensure that they obtain a report from Pest Contractor after each visit and retain all the records together with Form 1.

Chemicals used for Pest Control Management must be from an approved supplier (maybe supplied by the Pest Control Contractor) and the Material Safety Data Sheet (MSDS) must be kept at the facility in Appendix 3 Material Safety Data Sheets to this food safety program. All chemicals used for control of pests must be suitable and approved for use in a food premises and stored in a designated secured area away from food and food contact surfaces.

## 2.10 Customer Complaints

In this section, the program should outline on how the business receives and handles Customer complaints and conducts investigation to the cause of the complaint on Form 9.

Please ensure that the procedure is reviewed to the business’s needs.

The business shall keep a record of all customer complaints received by the business. Once a complaint has been raised, the employee receiving the complaint must complete Form 9 Customer Complaints with the following details:

- Complaint number;

- Name of complainant;

- Contact details of the complainant; and

- Details of the complaint (date of purchase, product, batch, reason for the complaint etc.).

The Operator/ Quality Assurance Manager will be notified immediately so that an initial assessment of the complaint can be made. The Operator/ Quality Assurance Manager decides whether to investigate the complaint and this can be dependent on a number of factors such as the severity of the complaint, the number of complaints received for the same product and whether there is a risk to public health. If the complaint poses a risk to public health, it must be reported to PrimeSafe. This notification may be in the form of a telephone call or email (info@primesafe.vic.gov.au).

As part of the complaint investigation process, the Operator/ Quality Assurance Manager must continue to complete Form 9 with further information regarding:

- Investigation details;

- Corrective actions that will be taken by the business; and

- Feedback to the complainant if requested.

If during the investigation, the complaint has further implications for potential health risks to many consumers i.e. broader public health, then the Operator/ Quality Assurance Manager must inform PrimeSafe and Department of Health and Human Services (DHS). The Operator/ Quality Assurance Manager must follow their Product Recall Procedure, which may include completing a Food Standard Code Australia and New Zealand (FSANZ) recall form. The recall process is managed by FSANZ; however, PrimeSafe will provide guidance to fulfil the recall obligations.

After the completion of the complaint investigation, the customer shall be notified of the findings and proposed resolution if required.

All complaints are recorded on Form 10 Customer Complaint Log and the customer complaints procedure must be reviewed annually as part of the internal audit program. Customer complaints must also be included as part of the Management Review. For customer complaints that pose a risk to health, a HACCP meeting must also take place to review all the processes to ensure continued compliance with Standards and to avoid recurrence of the issue.

In this section document information for Waste Management established in your organisation.

Waste Management is a vital part of minimising the food safety risk of cross contamination incident affecting the business.

The procedures should reflect the requirement of the relevant standard – Retail butcher shop (Victorian Standard for Hygiene Production of Meat at Retail Premise) and AS 4696 for processing high risk meat products such as Smallgoods).

2.11 Waste Management

The business has a waste disposal system to manage waste so that it does not jeopardise the wholesomeness of meat and meat products and to prevent an environmental nuisance from arising. To prevent contamination, food waste and other waste materials are removed from areas where food is processed.

The facility is maintained to a standard of cleanliness where waste does not accumulate and is disposed of with no risk or source of contamination to food products or the potable water supply. Waste holding areas are located where it is not near doors used for personnel or product entry into production areas or amenities.

The following are requirements for waste storage and removal:

- The storage facility for waste and inedible materials prevents infestation by pests and is capable of being effectively cleaned;

- An adequately sized grease trap and drainage system is installed and maintained to ensure it does not jeopardise the wholesomeness of meat and meat products and prevents blockages, odour problems and health hazards from occurring. For existing premises which do not have a grease trap, a written approval is required from the water authority;

- All catch basins, traps, save-alls, and sumps are separated from areas where meat and meat products are processed;

- Discharge is contained and directed to the drainage system; and

- Untreated waste from toilets is treated separately from facility waste and is not discharged into the waste from processing activities.

The operator and all personnel will do visual inspections throughout the day to ensure:

- Waste containers must be in good condition and have the capacity to contain the waste;

- Waste containers are emptied regularly so waste does not build up;

- Bins are emptied regularly and not overflowing (included in pre-operational checks);

- Waste storage areas are cleaned regularly (included in the cleaning schedule);

- Different types of waste containers are identifiable so they can be managed accordingly; and

- Cardboard boxes and packaging are flattened and placed in bins where garbage or recyclable matter is stored

## 2.12 Product Recall

A food recall is action taken by a food business to remove unsafe food from distribution, sale, and consumption. All food businesses must be able to quickly remove affected food from the marketplace to protect public health and safety. Food Standards Australia New Zealand (FSANZ) coordinates and monitors food recalls in Australia. PrimeSafe will work with the business to ensure a recall is conducted where required. See diagram below regarding responsibilities.

In this section document information for the Product Recall Program established in your organisation.

For further information on Food Recall, please visit Food Standards Australia New Zealand Website.

Recall or Withdrawal?

Food is recalled if there is a reasonable probability that the consumption of that food will cause adverse health consequence. For example, presence of bacteria such as Salmonella, E.coli, Coagulase Positive Staphylococcus, Listeria monocytogenes; and toxic chemical contaminants or harmful foreign bodies.

Reasons for food recall:

- microbial – contamination with pathogenic microorganisms such as bacteria, viruses or parasites;

- labelling – non-compliant labelling, incorrect food ingredients on the ingredient list, incorrect date markings or other food labelling errors;

- foreign matter – contamination with material such as glass, metal or plastic objects;

- chemical/other contaminants – contamination with substances such as cleaning products, pesticides, machine oil, etc.;

- undeclared allergen – due to incorrect labelling, incorrect packaging or contamination of the product by an allergen;

- packaging fault – where a fault in the food packaging results in contamination (presence of glass/metal etc.) of the food or a potential choking hazard;

- biotoxin – contamination with biological toxins such as histamine in fish and paralytic shellfish toxin in bivalve shellfish; and

- other – for example, unsafe levels of additives or presence of a therapeutic drug.

Withdrawal of a food occurs if the food is unlikely to cause adverse health consequences. For example, incorrect labelling, underweight or quality defect (texture or colour) or physical deterioration of the product. Withdrawal may also be conducted pending a further investigation of a potential food safety risk.

Responsibilities

The Operator/ Quality Assurance Manager (QAM) is responsible for conducting the recall and must make the decision to recall the product after consultation with the HACCP and/or Management teams of the business. The Operator/QAM must communicate with FSANZ and PrimeSafe and provide the following information:

- Nature of the problem/reason for the recall;

- Brand name and description of the product, including packaging size and type of packaging;

- Use or best before date;

- Lot identification;

- Quantity of the batch manufactured and the date and amount released;

- Quantity of the affected food product that can be accounted for;

- Distribution within Australia;

- Overseas distribution (if applicable); and

- Importer information (for imported products).

In the event of a recall, the system defined in the Food Recall Protocol prepared by FSANZ is used to conduct a product recall and prepare a food recall plan. Please ensure that the food recall protocols accessed are the most updated version from the FSANZ website.

Contacts

Local Authority Contact

PrimeSafe

Phone: (03) 9685 7333

Email: info@primesafe.vic.gov.au

Food Standards Australia New Zealand (FSANZ)

Phone: (02) 6271 2610 between 9am – 5pm Monday – Friday or 0412 166 965 after hours

## 2.13 Allergen Management

Some food ingredients used in meat processing can cause severe allergenic reactions including anaphylaxis. There are 10 common allergens listed in the Food Standards Code Australia New Zealand (FSANZ) that are linked to allergenic reactions and must be declared whenever they are present as ingredients or as components of food additives or processing aids:

Peanuts, tree nuts, milk, eggs, sesame seeds, fish, shellfish, soy, lupin, and wheat

If the food is not in a package or is not required to have a label (e.g. Food prepared and sold from a takeaway shop or food sold from a retail cabinet in a butcher shop), then this information must be either displayed in connection with the food or provided to the purchaser if requested.

When processing with allergens you should consider:

- Segregation of allergen containing foods, ingredients, and processing aids from allergen-free products;

- Processing allergen-free products before allergen-containing products;

- Storage of allergen containing products in lower racks than non-allergen containing products;

- Washing surfaces, equipment and utensils after processing products containing allergens;

- Swabbing surfaces for testing to ensure that they have been cleaned effectively;

In this section document information for Allergen Management established in your organisation.

Allergen Management is a vital part of minimising the food safety risk of undeclared allergen contamination incident affecting the business and consumers.

The procedures should reflect of how the business manage the allergens on site. Refer to Food Standards Code Australia New Zealand (FSANZ)

Colour coding equipment and utensils when multiple allergens are present on the premises;

- Labelling every product that contains allergens according to the requirements of the Food Standards Code – P1044 PEAL (Plain English) Allergen Labelling;

- Reviewing ingredient supplier specifications to assist in product labelling; and

- Use of common meat allergen such as Sulphur Dioxide, only used in sausages and sausage meat and not in fresh meat.

Responsibility

The Operator/ Quality Assurance Manager is responsible for the management of allergens and must ensure that it is included in the HACCP risk assessment. The HACCP team must also consider it in the internal audit review at least annually.

## 2.14 Product Identification and Traceability

All raw materials and finished products must be traceable throughout the supply chain and when it reaches the consumer.

All food products, raw materials and packaging materials received are recorded on Form 1 Weekly Diary. Relevant traceability records (i.e. food receipts, invoices, and delivery slips) where possible must be kept on file.

Meat products manufactured by the business must be identified with a batch number. The batch number may consist of production date and the batch number for the day or another identifier format. For example:

3112-01

[date, month, batch number for the day]

Rework and work in progress (WIP) products are to be clearly labelled with the following information:

- name of product;

- date when placed for rework; and

- batch number.

In this section, document information on the procedure for Product Identification and Traceability established in your organisation.

Product identification and traceability are vital for identifying products, allergens, and pathogens. This ensures that any product can be traced and removed from the market if a food safety issue is identified.

The procedures should reflect how the business manages product identification and traceability on site.

All packaged meat and meat products are required to have traceability through labelling. The label shall conform to the Australia New Zealand Food Standards Code Standard 1.2 – Labelling and other information requirements and Victorian Standard for Hygienic Production of Meat at Retail Premises.

Packaged meat product labels must have:

- description of the product and the species;

- batch number;

- name of business and its address;

- date of packaging and use by/best before date;

- storage conditions; and

- country of origin.

Packaged meat product that contain added ingredients labels must have:

- description of the product and the species;

- batch number;

- name of business and its address;

- date of packaging and use by/best before date; and

- storage conditions and direction of use;

- country of origin;

- Nutritional Information Panel;

- Ingredient listing;

- nutritional and health claims; and

- advisory statement, warning statement and declarations

The labelling check of packaged meat and meat products are recorded on Form 12 Labelling Assessment.

## 2.15 Document Control

Documents and records within the food safety program must be reviewed at least annually or whenever there is a change. All documents and records relating to the business must remain at the facility and stored appropriately. The documents and records require version and date control and must be accessible to nominated staff.

It is the responsibility of the operator or quality assurance manager (QAM) to ensure documents and records are:

- Adequate for use;

- Filled out correctly;

- Reviewed and updated when necessary; and

- Not using obsolete documents.

The business can customize their food safety program and request a change by contacting PrimeSafe. However, Form 2 Amendment Register must still be completed by the operator or/and QAM.

In this section, the document control procedure outlines how to maintain control of current documents. It provides guidance on managing updates and changes to documents such as HACCP plans, procedures, and monitoring forms, ensuring they are kept up to date and properly controlled.

Note: The purpose of this procedure is to prevent the creation of uncontrolled versions of documents that are not considered part of the food safety program.

This should reflect in Document Control Program established in your organisation.

All changes to the food safety program should be recorded on Form 2 Amendment Register.

Obsolete documents must be marked obsolete and appropriately archived to prevent unintended use.

Records to be maintained for a minimum of 3 months in accordance with Food Standards Code 3.2.2A.

Please ensure that all monitoring of CCPs and other prerequisites must be recorded to demonstrate compliance to the food safety program

# FORMS & MONITORING

Form 1 - Weekly Diary				Week Starting

| Monitoring Temperatures for Retail Display Section (≤5°C), Chillers (≤5°C), Freezers (approx ≤-15°C) All meat and smallgoods (<5°C); Carcass and quarter sides (≤7°C), |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mon |  | Tue |  | Wed |  | Thur |  | Fri |  | Sat |  | Sun |  | Corrective Actions Comments |
|  | AM | PM | AM | PM | AM | PM | AM | PM | AM | PM | AM | PM | AM | PM |  |
| Retail Display 1 CCP (°C) | 3.5 | 4.1 | 5.6 | 4.5 | 2.1 | 2.8 | Example 3.0 | 3.5 | 1.2 | 2.3 | 2.5 | 2.0 | 1.5 | 2.6 | Tue 10:30am – 5.6 over 5oC. Place back in chiller. Recheck temperature in 1 hour 4.8. (initial of checker) |
| Retail Display 2 CCP (°C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Chiller 1 CCP (°C) |  |  |  |  |  |  |  |  |  |  | Monitoring of temperature for each area is recorded twice daily when being used |  |  |  |  |
| Chiller 2 CCP (°C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Freezer CCP (°C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product 1 CCP (°C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product 2 CCP (°C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product In-Process CCP (°C) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Pre-operational checks before processing, display and sale |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Daily | Mon | Tue | Wed | Thur | Fri | Sat | Sun |  |
| Hand Wash Basins |  |  |  |  |  |  |  |  |
| Floors/ Walls/ Ceiling/ Doors |  |  |  |  |  |  |  |  |
| Benches/ Tables/ Cutting Boards/ Scales |  |  |  |  |  |  |  | Mon - Table had meat stuck on the leg – reclean |
| Tools/ Equipment/ Tubs/ Trays |  |  |  | Example |  |  |  |  |
| Display Cabinets |  |  |  |  |  |  |  |  |
| Chillers/ Freezers |  |  |  |  |  |  |  | Mon -Boxed meat on the floor – remove all boxes off the floor. |
| Waste Bins |  |  |  |  |  |  |  | Mon - Waste bin dirty – re-washed. |
| Amenities |  |  |  |  |  |  |  |  |
| Pest Activity Sighted |  |  |  |  |  |  |  |  |
| Meat Transport Vehicle (MTV) | N/A | N/A | N/A | N/A | N/A | N/A | N/A | No MTV |
| Staff clothing/ aprons, hats, gloves |  |  |  |  |  |  |  |  |
| Staff washing hands regularly after contamination, smoking, toilet |  |  |  |  |  |  |  |  |
| Segregation of raw & cooked products |  |  |  |  |  |  |  |  |
| Weekly |  |  |  |  |  |  |  |  |
| Dry Storage Room |  |  |  |  |  |  |  |  |
| Chemical Storage Area |  |  |  |  |  |  |  |  |
| Fans, Exhaust & Lights |  |  |  |  |  |  |  | Mon - Fan covered in dust – dissembled and clean |

Authorised Food Safety Monitor   Name: _______________     Signature: _____________     Date: ______________

Product Receival & Dispatch 	                       *(more receival, dispatch and corrective actions can be recorded over the page)

| Receival Date | Product Description | Quantity | Supplier | Temp (°C) | MTV Licence No. |
| --- | --- | --- | --- | --- | --- |
| 13/08/2024 | Boxed beef brisket | 20kg | Example Beef company | 3.5 | 07051 |
| 13/08/2024 | Chicken wings | 10kg | Chicken Ready | 5.6 | 07032 |
| Dispatch Date | Product Description | Quantity | Customer | Temp (°C) | Batch Number |

Corrective Action Report -continued

| Date | Issue | Date Fixed | Signature- Food Safety |
| --- | --- | --- | --- |
| 13/08/2024 | High temperature of chicken wings received. Place chicken wings immediate in chiller and monitor temperature 30 mins later. Temperature recheck 5.0*C | Example 13/08/2024 | Initial here |

Authorised Food Safety Monitor   Name: _______________     Signature: ___________   Date: ______________

Updating your food safety program should always be recorded to ensure document control is maintained and current version is used.

Form 2 - Amendment Register

| Date | Version | Document | Amendment | Signature |
| --- | --- | --- | --- | --- |
| 04/8/24 | 02 | Example Form 1 - Weekly Diary | Additional equipment for temperature monitoring – Freezer | Initial here |

| Date | Equipment | Serial No. | Method | Set Reading | Measured Reading | Difference | Corrective Action / Out of Specification | Signed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13/08/24 | Handheld Thermometer # 1 | 2336524 | Hot water | 100 | 99.5 | 0.5 |  | J. D |
| 13/08/24 | Handheld Thermometer # 2 | 2335547 | Example Hot water | 100 | 90 | 10 | Discard and change with new thermometer serial # 2333355 | J. D |
|  |  |  |  |  |  |  | Calibration should be recorded on this register to ensure that equipment used is in working order – Follow your 2.1 Calibration procedure. |  |

## Form 3 - Calibration Register

## Form 4 - Monthly Maintenance and Repair

| Monthly | Comment/ Non-conformance | Corrective Action | Date Completed | Signed |
| --- | --- | --- | --- | --- |
| Floors - free from cracks and damage; Coving – free from damage & gaps |  |  |  |  |
| Walls – free from damage, mould, peeling paint etc. |  |  |  |  |
| Ceiling – free from damage, mould, peeling paint etc. |  |  | Maintenance should be recorded on this register on a monthly frequency to ensure that construction and equipment used fit for purpose and in working order – Follow your 2.2 Maintenance procedure. |  |
| All equipment free from damage, rust and corrosion |  |  |  |  |
| All processing implements Boards, knives etc. free from damage, rust and corrosion |  |  |  |  |
| Hand-wash sink/ sinks Accessible, in working order Used as fit for purpose (Temperature: 35-46°C) |  |  |  |  |
| Overhead lighting Lights covered over process area, undamaged |  |  |  |  |
| Food & Equipment Storage Areas tidy, free from damage and pest activity |  |  |  |  |
| Display Units, Chillers & Freezers In working order, cooling units have no ice build-up, doors operational and seals undamaged, no mould. Used as fit for purpose. |  |  |  |  |
| Food & Equipment Storage Areas tidy, free from damage and pest activity |  |  |  |  |
| Food Packaging Covered & stored to prevent contamination |  |  |  |  |
| Chemicals Stored correctly and separate from food |  |  |  |  |
| Staff Amenities Clean, tidy and free from pest activity and allergens |  |  |  |  |
| Pest Monitoring Stations Undamaged and correctly positioned |  |  |  |  |
| Meat Transport Vehicle Free of damage to doors, floors, walls & railings. Free of rust, corrosion & peeling paint. Cooling unit in working order. |  |  |  |  |

| Product | Supplier | Supplier Address | Contact Details (Name & Number) | Comments |
| --- | --- | --- | --- | --- |
|  | This should list the approved suppliers (including suppliers of product, packaging and ingredients) that provide safe foods to your organisation. Suppliers should be approved based on timely delivery, adherence to the organisation's specifications, and food safety. Follow your 2.3 Approved Supplier procedure. |  |  |  |

## Form 5 - Approved Suppliers List

## Form 6 – Staff Training Register

| Date | Staff Member | Type of Training | Trained By: | Staff Signature | Trainer Signature |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Staff who handles food at the organisation should be trained on food safety against the procedures in this food safety program. Note: that any external visitors to the site shall be trained against the food safety procedures, You can amend this section to include training for external visitors and/ or contractors that attend at your premise. Training is recorded on this register, and a refresher training should be conducted in accordance – Follow your 2.7 Staff Training procedure. |  |  |

## Form 7 - Induction Training

Name:

Position:

Date:

The following information below has been discussed with the new employee:

Welcome and Facility Tour:

New employees who are hired that handle food at the organisation should be inducted and trained on food safety against the procedures in this food safety program. Training is recorded on this register, and a refresher training should be conducted in accordance – Follow your 2.7 Staff Training procedure.

Note: This page may be amended to include external visitors in the induction training, ensuring compliance with the food safety program.

□ Introducing new employee to staff

□ Safety in the workplace

□ Payroll

□ Emergency Exits

□ Processing Floor

□ Tea Room / Kitchen and Rest Room

Food Safety Procedures:

□ Cleaning and Sanitation

□ Good Manufacturing Practices

□ Personal Hygiene Requirements

□ Allergen Management

□ Waste Management

Trainee Signature 						Trainer Signature

____________________					____________________

Trainee Name:						Trainer Name:

____________________					___________________

## Form 8 - Cleaning & Sanitation Schedule

| Area | Frequency |
| --- | --- |
| Retail Display |  |
| Tools/Equipment | Daily |
| Benches, Tables, Cutting Board, Tubs, Trays, Utensils | Daily |
| Scales | Daily |
| Floors, Walls, Ceilings, Doors | Daily |
| Display Units | Daily |
| Counter | Daily |
| Hand Wash Basin | Daily |
| Waste Bins | Daily |
| Register | Weekly |
| Processing |  |
| Hand Wash Basin | Daily |
| Benches, Tables, Cutting Board, Tubs, Trays, Utensils | Daily |
| Floors/Walls/ Ceilings/Doors | Daily |
| Tools/ Equipment | Daily |
| Waste Bins | Daily |
| Fans | Weekly |
| Other |  |
| Meat Transport Vehicle (MTV) | Daily |
| Dry Storage Room (Floors, walls, ceilings, doors, racking) | Weekly |
| Chemical Storage Room (Floors, walls, ceilings, doors, racking) | Weekly |
| Chillers (Floors, walls, ceilings, doors, racking) | Weekly |
| Freezer (Floors, walls, ceilings, doors, racking) | Weekly |
| Fans and Exhaust | Weekly |
| Pest Control Stations | Weekly |
| Amenities | Weekly |
| Staff Clothing, aprons, hats, gloves | Weekly |

| Chemical | Purpose/Detergent Sanitiser | Dosage |
| --- | --- | --- |
|  |  | The cleaning and sanitation schedule should list the construction items and equipment that need to be cleaned and sanitised at specified frequencies. Follow your 2.8 Cleaning and Sanitation procedure |

## Form 9 – Customer Complaints

| Date of Complaint: | Complaint Number: |
| --- | --- |
| Complainant Details: Name: Contact Details: |  |
| Details of Complaint |  |
| All Customer complaints should be recorded and handled accordingly of investigation the cause of the complaint. Follow your 2.10 Customer Complaint procedure Investigation Details |  |
| Corrective Actions |  |
| Preventative Actions |  |
| Complaint Closed: Name: Signature & Date: |  |

## Form 10 – Customer Complaint Log

| Complaint No. | Date of Complaint | Issue Type | Closed Out Date | Signed |
| --- | --- | --- | --- | --- |
| 1 (Example) | 09/07/2018 | Foreign material found in beef mince | 16/07/18 | A.Z |
|  | All Customer complaints should be logged into the Complaint log. Follow your 2.10 Customer Complaint procedure |  |  |  |

## Form 11 – Meat Transport Vehicle (MTV) Monitoring and Maintenance

| Vehicle Registration No: ________________________ | Month & Year: _______________________ |
| --- | --- |

| Date | Meat/Seafood carrying compartment is clean prior to loading meat and/or seafood products. (Y/N) | Temperature of meat and/or seafood product (5°C or below) | Temperature of vehicle for transport of meat only (5°C or below) | Signature |
| --- | --- | --- | --- | --- |
| 24/08/18 | Y | 4°C | 4°C | A.Z |
|  |  |  | All meat and seafood fit for human consumption should be transported in a licensed vehicle. Ensure temperature of the product is recorded and that the construction of the carrying compartment is compliant to AS 4696. Follow your 2.2 Maintenance Program and 2.6 Good Manufacturing Practices |  |
| Maintenance: Details of structural and mechanical defects identified in meat/seafood carrying compartment. |  |  |  |  |

## Form 12.1 – Labelling Assessment for Packaged Meat Products

| Date | Product | Batch No. | Quantity | Labelling Checklist (✓) |  |  |  |  | Signed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Description of product and the species | Name of business and its address | Date of packaging and use by/best before date | Storage conditions | Country of origin |  |
|  |  |  |  |  |  |  |  |  | Labelling assessment (Form 12.1 & 12.2) for packaged meat and meats products assists in identification, traceability and allergen declaration. Follow your 2.14 Product Identification and Traceability |
|  | [Insert rows] |  |  |  |  |  |  |  |  |

## Form 12.2 – Labelling Assessment for Packaged Meat Products with Added Ingredients

| Date | Product | Batch No. | Quantity | Labelling Checklist (✓) |  |  |  |  |  |  |  |  | Signed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Description of product and the species | Name of business and its address | Date of packaging and use by/best before date | Storage conditions and direction of use | Country of origin | Nutritional Information Panel | Ingredient listing (including allergens) | Nutrition and health claims | Advisory/ Warning statements and declaration |  |
|  |  |  | [Insert rows] |  |  |  |  |  |  |  |  |  |  |

Processing sausages with preservatives such as Sulphur Dioxide (SO2) on site will require the organisation to declare the use. Use this register to record all incoming batch of preservatives used on site.

Form 13 – Declaration and Use of Preservatives

| SECTION A: NO PRESERVATIVE USE ON SITE This declaration is made by the operator of this business that no preservative (Sulphur Dioxide) is used at this premises. I understand that PrimeSafe will install appropriate sanctions if this procedure is breached. ________________ ________________ ________________ Operator Signature Date | SECTION B: PRESERVATIVE USE ON SITE Preservative (Sulphur Dioxide) is used at this premises in accordance with the Food Standards Australia New Zealand (FSANZ). Sulphur Dioxide is not permitted to be added in any other products except sausages, where only 500mg/kg is permitted. The use of preservative is monitored by the following persons and recorded in the register below. ________________ ________________ ________________ Name Signature Date ________________ ________________ ________________ Name Signature Date |
| --- | --- |

| Date | Product | Batch No. | Quantity of Meat (kg) | Sausage Mix/ Sulphur Dioxide | Quantity of Preservative (g or ml) | Signed |
| --- | --- | --- | --- | --- | --- | --- |

Example: PRESERVATIVE REGISTER

# APPENDIX

## Appendix 1 - Calibration

Scales

- Level the scale using its adjustable feet. Ensure the bubble in the bubble-level is within the circle.

- Tare the scales so display reads at zero.

- Place a food-grade known certified weight onto the scales.

Hand-held Thermometer

Ice Water Slurry Method

- Fill a container with crushed ice (50g-100g)

- Add water to the container and stir. (Note: Enough to produce a slurry but not so much that the ice floats)

- Let it stand for 2 minutes.

- Place the hand-held thermometer in the centre of the container.  The probe should not touch the base of the container.

- Allow the hand-held thermometer to reach a steady reading. The reading should be 0°C.

- Repeat steps 1-5 twice and at least one minute apart.

Boiling Water Method:

- Fill a saucepan/pot with water and bring it to the boil. Ensure the water can maintain at boiling point throughout the calibration process.

- Place the hand-held thermometer in the centre of the saucepan/pot. The probe should not touch the bottom of the saucepan/pot.

- Allow the hand-held thermometer to reach a steady reading. The reading should be 100°C.

- Repeat steps 1-3 twice and at least one minute apart.

Fixed Thermometer

- Place a calibrated hand-held thermometer that has been calibrated with Ice Water Slurry Method as close as possible to the gauge probe for at least 2 minutes. Ensure the door is not opened during the calibration. This will allow the hand-held thermometer to reach a steady reading.

- Repeat step 1 twice and at least one minute apart.

- Compare the reading on the fixed thermometer against the calibrated hand-held thermometer.

## Appendix 2 – Pest Control Station Map

## Appendix 3 – Determining food safety risk – CCP Decision Tree

Q1. Are preventive measures in place?

Q2. Do the measures reduce the hazard?

Q3. Could hazards reach unacceptable levels?

Q4. Will a subsequent step reduce or eliminate the hazard?

Q5. Is control necessary?

| Process step | Hazard | Control Measure | Q1 | Q2 | Q3 | Q4 | Q5 | CCP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

CCP = Critical control point 	PR = Prerequisite Program

## Appendix 4 - Material Safety Data Sheets (MSDS)